# Code documentation:

AVR121 code documentation can be found in the subfolder '\doxygen\html\’. Open the file ‘index.html’ to access the documentation.

Index.html refers to two different source-code examples. Both examples use a counter to produce an artificial noise signal, and utilizes this signal to enhance the ADC resolution. In cases where the environmental noise is sufficient to toggle the LSB of the conversion result, this segment could be removed from the source-code. Both examples uses interrupt driven ADC.

'main_uart.c' has a UART interface, and is meant as an educational example, where the user can verify that the resolution really increases. To log the samples the free software terminal emulator, Tera Term Pro was used. Though other terminal programs may be just as useful.

'main.c' is a stripped example meant for usage in an field-application. It is recommended to remove/modify the error compensation in both examples; this is left behind in the code to remind the user that it most cases it is necessary with some kind of error compensation, in accordance to Application Note AVR120.